BRRI Winnower

| Section | Item | Specification |
| --- | --- | --- |
| General information | Design | BRRI |
|  | Model | BRRI Win2024 |
|  | Country of origin | Bangladesh |
|  | Country of manufacture | Bangladesh |
|  | Classification | Multicrop winnower machine |
|  | Application | Cleaning and separation of grains from chaff and impurities |
|  | Operation type | Electrically operated |
| Dimensions and weight | Overall dimension (L×W×H), mm | 1220×815×1330 |
|  | Motor base (L×W), mm | 395×197 |
|  | Machine weight, kg | 115 |
| Power unit | Motor type | Electric motor |
|  | Motor specification | Single phase |
|  | Motor power, hp | 0.5 |
|  | Motor speed, rpm | 1400-1450 |
|  | Power transmission | Belt and pulley drive |
| Feeding unit | Grain hopper dimension (L×W×H), mm | 620×580×320 |
|  | Feeding method | Manual feeding |
|  | Load capacity, kg | 450–550 |
| Separation unit | Separation method | Air blowing and vibration mechanism |
|  | Vibrating plate length, mm | 237 |
|  | Eccentric linkage length, mm | 225 |
|  | Wind board dimension (L×W), mm | 483×228 |
|  | Cleaning mechanism | Combined airflow and oscillation |
| Sieve mesh size | For mustard, mm | 6 |
|  | For paddy, mm | 10 |
|  | For maize, mm | 13 |
| Performance | Winnowing capacity, kg/hr | 700-800 |
| Accessories |  | Three interchangeable sieves for crop-specific winnowing of mustard, paddy, and maize. |

Q1  ·  To fix the highlight, we need:

Cleaning capacity

Official kg per hour?

Answer: 700-800 kg/h (official stated winnowing capacity).

Q2  ·  To fix the highlight, we need:

Time for 2 acres

Official time?

Answer: আনুমানিক ৪–৬ ঘণ্টা সময় লাগতে পারে। BRRI Winnower-এর কার্যক্ষমতা প্রায় ৭০০–৮০০ কেজি/ঘণ্টা। ২ একর জমি থেকে আনুমানিক ৩–৪ টন ধান পাওয়া গেলে তা পরিষ্কার করতে প্রায় ৪–৬ ঘণ্টা লাগবে। তবে ফলন, ধানের আর্দ্রতা, ময়লার পরিমাণ ও ফিডিং রেটের ওপর সময় কিছুটা কম-বেশি হতে পারে।

Q3  ·  To fix the highlight, we need:

Labour saved

Official hours or taka?

Answer: আনুমানিক ৭০–৮০% শ্রমিকের প্রয়োজন কমতে পারে; ৪–৫ জনের পরিবর্তে সাধারণত ১–২ জন শ্রমিক দিয়ে কাজ পরিচালনা করা সম্ভব।

Q4  ·  To fix the highlight, we need:

Official name

Confirm: ব্রি উইনোয়ার / ব্রি ধান গম ঝাড়াই যন্ত্র?

Answer: Official name: BRRI Multicrop Winnower; model: BRRI Win2024. ব্রি শস্য ঝাড়াই যন্ত্র

Q5  ·  To fix the highlight, we need:

Hopper photo

Which official hopper photo to show?

Answer:

Q6  ·  To fix the highlight, we need:

Screen word

Always চালনি / sieve — never জাল?  (yes / no)

Answer: Yes. Use ‘chaluni’ in Bangla and ‘sieve’ in English; do not use ‘jal’.

Q7  ·  To fix the highlight, we need:

Screen word

Always চালনি — never ঝরনি?  (yes / no)

Answer: Yes. Use ‘chaluni’; do not use ‘jharani’.

Q8  ·  To fix the highlight, we need:

Frame size

Confirm 38×38×3?

Answer: confirmed. 38×38×3 mm

Q9  ·  To fix the highlight, we need:

Official name

Confirm: ব্রি উইনোয়ার / ব্রি ধান গম ঝাড়াই যন্ত্র?

Answer: Official name: BRRI Multicrop Winnower; model: BRRI Win2024.

Q10  ·  To fix the highlight, we need:

Frame material

Confirm mild steel?  (yes / no)

Answer: yes

Q11  ·  To fix the highlight, we need:

Name and × sign

Official name? Write sizes with × not গুণ?

Answer: Use the official name ‘BRRI Multicrop Winnower (BRRI Win2024)’ and write dimensions with the multiplication sign (×), e.g., 1220×815×1330 mm.

Q12  ·  To fix the highlight, we need:

Material and × sign

Confirm mild steel angle bar, 38×38×3?

Answer: yes

Q13  ·  To fix the highlight, we need:

Gusset

Official part? If yes: size and drawing?

Answer:

Q14  ·  To fix the highlight, we need:

Complete assembly drawing

Official file or sheet name?

Answer:

Q15  ·  To fix the highlight, we need:

Exploded-view drawing

Official file (readable labels)?

Answer:

Q16  ·  To fix the highlight, we need:

Shaft size and name

Diameter and length (mm)? Official name?

Answer:

Q17  ·  To fix the highlight, we need:

Hopper CAD

Official sheets to show?

Answer:

Q18  ·  To fix the highlight, we need:

BOM file

Part, quantity, material, size?

Answer:

Q19  ·  To fix the highlight, we need:

Connection drawing

Official file or sheet?

Answer:

Q20  ·  To fix the highlight, we need:

Screen word

Always চালনি / sieve — never ঝরনি?  (yes / no)

Answer: Yes. Use ‘chaluni’ in Bangla and ‘sieve’ in English; do not use ‘jharani’.

Q21  ·  To fix the highlight, we need:

Capacity figures

Official current kg/h? Target kg/h?

Answer: বর্তমান ক্ষমতা ৭০০–৮০০ কেজি/ঘণ্টা। ১,০০০ কেজি/ঘণ্টা লক্ষ্য অর্জনের জন্য মূলত ফিডিং রেট, চালনির ক্ষেত্রফল ও গতি, ব্লোয়ারের বায়ুপ্রবাহ এবং প্রয়োজন অনুযায়ী মোটর ও পাওয়ার ট্রান্সমিশন সিস্টেমের ক্ষমতা বাড়ানো/অপটিমাইজ করা প্রয়োজন